\documentclass[conference]{IEEEtran}

% =========================================================

% PACKAGES

% =========================================================

\usepackage{amsmath}

\usepackage{amssymb}

\usepackage{graphicx}

\usepackage{cite}

\usepackage{booktabs}

\usepackage{array}

\usepackage{url}

\usepackage{float}

\usepackage{siunitx}

\usepackage{multirow}

% =========================================================

% DOCUMENT

% =========================================================

\begin{document}

% =========================================================

% TITLE

% =========================================================

\title{Kalman Filter Signal Denoising}

% =========================================================

% AUTHOR

% =========================================================

\author{

\IEEEauthorblockN{K Sai Sushanth Reddy , Dr.D.Bhasker}

\IEEEauthorblockA{

Department of Electronics and Communication Engineering\\

Chaitanya Bharathi Institute of Technology\\

Hyderabad, Telangana, India\\

Email: bhaskerd\_ece@cbit.ac.in , saisushanthreddykssr2007@gmail.com

}

}

\maketitle

% =========================================================

% ABSTRACT

% =========================================================

\begin{abstract}

Noise is present in most practical digital signals and can make it harder to recover the information they carry. This project explores how a Kalman filter can be used to estimate a signal affected by noise. A discrete-time signal is created in MATLAB and mixed with additive Gaussian noise to represent noisy measurements. The filter then uses a state-space model to predict the signal and refine that prediction whenever a new measurement is received. Its performance is examined in the time and frequency domains using Mean Square Error (MSE), Root Mean Square Error (RMSE), and Signal-to-Noise Ratio (SNR). The results are used to assess how well the chosen model and covariance settings work, and to identify where the current implementation needs improvement.

\end{abstract}

% =========================================================

% KEYWORDS

% =========================================================

\begin{IEEEkeywords}

Kalman filter, signal denoising, digital signal processing,

state-space model, Gaussian noise, MSE, RMSE, SNR, MATLAB.

\end{IEEEkeywords}

% =========================================================

% I. INTRODUCTION

% =========================================================

\section{Introduction}

In many digital signal processing applications, the signal received by a system is not perfectly clean. Noise can come from electronic components, sensors, communication channels, environmental interference, or quantization. These unwanted variations can hide useful information, so a key task is to recover an estimate of the original signal without losing its important features.

The Kalman filter is a recursive method for estimating the state of a changing system when measurements are uncertain \cite{kalman1960}. It describes the system with a state-space model and combines a prediction from that model with each new measurement. This makes it possible to update the estimate as data arrives, rather than processing the entire signal in one step.

Kalman filters are used in areas such as navigation, robotics, control, communications, radar, tracking, and digital signal processing. One useful feature is that the estimate can be updated continuously as each new measurement becomes available.

The algorithm has two main stages: prediction and correction. First, the previous estimate is used to predict the next state. The filter then compares this prediction with the new measurement and adjusts the estimate. The Kalman gain controls how strongly the measurement influences that adjustment.

For this project, MATLAB is used to generate a discrete-time signal made up of three sinusoidal components. Gaussian noise is added at a specified input SNR, and a two-state Kalman filter is applied. The two states represent the signal amplitude and its rate of change. MSE, RMSE, and SNR are then used to evaluate the result.

The aim is not simply to implement the algorithm, but also to understand how the chosen state-space model and covariance values affect the quality of the estimate.

% =========================================================

% II. PROBLEM FORMULATION

% =========================================================

\section{Problem Formulation}

Let the original discrete-time signal be represented by x[k].

The measured signal is assumed to be corrupted by additive Gaussian

noise:

\begin{equation}

y[k] = x[k] + v[k]

\end{equation}

where

\begin{itemize}

\item x[k] is the original signal,

\item y[k] is the noisy measurement,

\item v[k] is additive Gaussian noise.

\end{itemize}

The goal is to recover an estimate of the original signal from these noisy measurements.

The estimation problem can be represented in state-space form as

\begin{equation}

X[k] = F X[k-1] + w[k]

\end{equation}

and

\begin{equation}

Z[k] = H X[k] + v[k]

\end{equation}

where X[k] is the system state, F is the state transition matrix,

H is the measurement matrix, w[k] represents process noise, and

v[k] represents measurement noise.

Because the original signal is known in this simulation, the estimate can be compared directly with it to measure the filter's performance.

% =========================================================

% III. SIGNAL AND NOISE MODEL

% =========================================================

\section{Signal and Noise Model}

\subsection{Sampling Parameters}

The sampling frequency used in the simulation is

\begin{equation}

F_s = 1000~\mathrm{Hz}

\end{equation}

and the sampling period is

\begin{equation}

T = \frac{1}{F_s}.

\end{equation}

A total of 5000 samples are generated. Therefore, the total simulation

duration is approximately 5 seconds.

The simulation parameters are summarized in Table~\ref{tab:parameters}.

\begin{table}[H]

\centering

\caption{Simulation Parameters}

\label{tab:parameters}

\begin{tabular}{lc}

\toprule

\textbf{Parameter} & \textbf{Value} \\

\midrule

Sampling Frequency & 1000 Hz \\

Number of Samples & 5000 \\

Simulation Duration & 5 s \\

Input SNR & 5 dB \\

Frequency f1 & 100 Hz \\

Frequency f2 & 250 Hz \\

Frequency f3 & 400 Hz \\

Amplitude A1 & 1.0 \\

Amplitude A2 & 0.5 \\

Amplitude A3 & 0.3 \\

\bottomrule

\end{tabular}

\end{table}

\subsection{Original Signal}

The original signal is composed of three sinusoidal components:

\begin{equation}

x(t) =

\sin(2\pi f_1t)

+

0.5\sin(2\pi f_2t)

+

0.3\sin(2\pi f_3t)

\end{equation}

where

\begin{equation}

f_1 = 100~\mathrm{Hz},

\end{equation}

\begin{equation}

f_2 = 250~\mathrm{Hz},

\end{equation}

and

\begin{equation}

f_3 = 400~\mathrm{Hz}.

\end{equation}

Combining these three frequencies gives a multi-component signal that can be used to test the estimation method.

\subsection{Gaussian Noise}

Zero-mean Gaussian noise is generated using MATLAB's \texttt{randn}

function.

The required noise power is calculated from the desired input SNR:

\begin{equation}

P_{\mathrm{noise}}

=

\frac{P_{\mathrm{signal}}}

{10^{\mathrm{SNR}_{\mathrm{input}}/10}}.

\end{equation}

The noisy signal is then obtained as

\begin{equation}

y[k] = x[k] + v[k].

\end{equation}

For this experiment, the target input SNR is set to 5 dB.

% =========================================================

% IV. KALMAN FILTER METHODOLOGY

% =========================================================

\section{Kalman Filter Methodology}

A Kalman filter estimates a system's state by combining what its mathematical model predicts with the information in noisy measurements. As described in \cite{kalman1960,brown2012}, each update consists of a prediction stage followed by a correction stage.

\subsection{State Vector}

In this implementation, a two-dimensional state vector is used:

\begin{equation}

X[k] =

\begin{bmatrix}

x[k]\\

\dot{x}[k]

\end{bmatrix}

\end{equation}

where the first state represents the signal amplitude and the second

state represents its rate of change.

\subsection{State Transition Matrix}

The state transition matrix is defined as

\begin{equation}

F =

\begin{bmatrix}

1 & T\\

0 & 1

\end{bmatrix}.

\end{equation}

With this matrix, the next amplitude is predicted using the current amplitude and its estimated rate of change.

\subsection{Measurement Matrix}

Only the signal amplitude is directly measured. Therefore,

\begin{equation}

H =

\begin{bmatrix}

1 & 0

\end{bmatrix}.

\end{equation}

The measurement equation is consequently

\begin{equation}

Z[k] = H X[k] + v[k].

\end{equation}

\subsection{Process Noise Covariance}

The process noise covariance matrix is selected as

\begin{equation}

Q =

\begin{bmatrix}

0.001 & 0\\

0 & 0.001

\end{bmatrix}.

\end{equation}

The matrix Q describes the uncertainty associated with the system model.

\subsection{Measurement Noise Covariance}

The measurement noise covariance is selected according to the

calculated noise power:

\begin{equation}

R = P_{\mathrm{noise}}.

\end{equation}

The value of R represents the assumed level of uncertainty in the measurements.

% =========================================================

% V. KALMAN FILTER ALGORITHM

% =========================================================

\section{Kalman Filter Algorithm}

The filter processes the signal one sample at a time. For each new measurement, it first predicts the next state and then uses the measured value to correct that prediction.

\subsection{Prediction Step}

The predicted state is calculated as

\begin{equation}

X_{k|k-1} = F X_{k-1|k-1}.

\end{equation}

The predicted covariance is

\begin{equation}

P_{k|k-1}

=

F P_{k-1|k-1}F^T + Q.

\end{equation}

\subsection{Kalman Gain}

The Kalman gain is calculated using

\begin{equation}

K_k =

P_{k|k-1}H^T

\left(

HP_{k|k-1}H^T + R

\right)^{-1}.

\end{equation}

The Kalman gain controls the relative contribution of the predicted

state and the noisy measurement.

\subsection{Correction Step}

The measurement residual is

\begin{equation}

r_k =

Z_k - H X_{k|k-1}.

\end{equation}

The updated state is then

\begin{equation}

X_{k|k}

=

X_{k|k-1}

+

K_k r_k.

\end{equation}

The updated covariance matrix is

\begin{equation}

P_{k|k}

=

(I-K_kH)P_{k|k-1}.

\end{equation}

The first state of the updated vector is stored as the Kalman

estimated signal.

% =========================================================

% VI. MATLAB IMPLEMENTATION

% =========================================================

\section{MATLAB Implementation}

The complete simulation was implemented in MATLAB. The main stages

of the implementation are:

\begin{enumerate}

\item Define sampling frequency and number of samples.

\item Generate the original multi-frequency signal.

\item Calculate the required Gaussian noise power.

\item Add Gaussian noise to the original signal.

\item Initialize the Kalman filter state and covariance matrices.

\item Perform prediction and correction for every sample.

\item Store the estimated signal and rate-of-change state.

\item Calculate MSE, RMSE, and SNR.

\item Generate time-domain and frequency-domain plots.

\end{enumerate}

Because the algorithm is recursive, it can update its estimate sample by sample without needing the full signal beforehand.

% =========================================================

% VII. TIME DOMAIN RESULTS

% =========================================================

\section{Time-Domain Results}

Fig.~\ref{fig:time_domain} compares the original signal with the noisy input and the signal estimated by the Kalman filter.

\begin{figure}[H]

\centering

\includegraphics[width=\columnwidth]{kalman_time_domain.eps}

\caption{Time-domain comparison of original, noisy, and Kalman-estimated signals.}

\label{fig:time_domain}

\end{figure}

The original waveform is the reference signal. Random variations appear in the noisy waveform because Gaussian noise has been added, while the Kalman output shows the estimate produced by the selected two-state model.

The estimate follows some of the signal's overall behavior, but the current settings do not improve the reconstruction error. In fact, the numerical results show that the filtered signal is less accurate than the noisy input.

% =========================================================

% VIII. ZOOMED TIME DOMAIN

% =========================================================

\section{Zoomed Time-Domain Analysis}

To examine the filtering behavior more closely, the first 500 samples

are plotted separately in Fig.~\ref{fig:zoomed}.

\begin{figure}[H]

\centering

\includegraphics[width=\columnwidth]{kalman_zoomed.eps}

\caption{Zoomed time-domain comparison over the first 500 samples.}

\label{fig:zoomed}

\end{figure}

Viewing the first 500 samples makes it easier to compare the three waveforms closely. This view also shows how the filter responds to the rapid variations in the signal.

The model tracks only amplitude and rate of change, so it may be too simple to describe a signal made up of several sinusoidal components.

% =========================================================

% IX. FREQUENCY DOMAIN ANALYSIS

% =========================================================

\section{Frequency-Domain Analysis}

Looking at the signal in the frequency domain helps show which frequency components remain after filtering and which may be reduced.

The discrete Fourier transform is calculated using MATLAB's

\texttt{fft} function:

\begin{equation}

X(f) = \mathrm{FFT}\{x[k]\}.

\end{equation}

Similarly, the noisy and estimated signals are transformed as

\begin{equation}

Y(f) = \mathrm{FFT}\{y[k]\}

\end{equation}

and

\begin{equation}

K(f) = \mathrm{FFT}\{x_{\mathrm{Kalman}}[k]\}.

\end{equation}

The magnitude spectra are calculated as

\begin{equation}

|X(f)| =

\frac{|\mathrm{FFT}(x[k])|}{L}.

\end{equation}

The resulting frequency-domain comparison is shown in

Fig.~\ref{fig:frequency}.

\begin{figure}[H]

\centering

\includegraphics[width=\columnwidth]{kalman_frequency_spectrum.eps}

\caption{Frequency spectra of original, noisy, and Kalman-estimated signals.}

\label{fig:frequency}

\end{figure}

The original spectrum has its main components at 100 Hz, 250 Hz, and 400 Hz. In the noisy signal, Gaussian noise adds energy across a wider range of frequencies.

Comparing the estimated spectrum with the original one helps identify which signal components the filter preserves and which it attenuates.

% =========================================================

% X. KALMAN STATE ESTIMATE

% =========================================================

\section{Kalman State Estimation}

The second state of the Kalman filter represents the estimated rate of

change of the signal. This state is stored during the filtering

process and plotted in Fig.~\ref{fig:state}.

\begin{figure}[H]

\centering

\includegraphics[width=\columnwidth]{kalman_state_estimate.eps}

\caption{Estimated rate of change obtained from the Kalman state vector.}

\label{fig:state}

\end{figure}

The second state gives an estimate of how quickly the signal is changing. It is updated recursively along with the amplitude estimate.

This is one advantage of a state-space model: it can estimate quantities that are not measured directly.

% =========================================================

% XI. PERFORMANCE EVALUATION

% =========================================================

\section{Performance Evaluation}

The filter is evaluated using three numerical measures:

\subsection{Mean Square Error}

The Mean Square Error is defined as

\begin{equation}

MSE =

\frac{1}{L}

\sum_{k=1}^{L}

\left(x[k]-\hat{x}[k]\right)^2

\end{equation}

where x[k] represents the estimated signal.

A smaller MSE means that, on average, the estimate differs less from the original signal.

\subsection{Root Mean Square Error}

The Root Mean Square Error is calculated as

\begin{equation}

RMSE = \sqrt{MSE}.

\end{equation}

RMSE is expressed in the same units as the signal amplitude and gives an intuitive measure of estimation error.

\subsection{Signal-to-Noise Ratio}

The effective SNR used in the MATLAB analysis is calculated as

\begin{equation}

SNR =

10\log_{10}

\left(

\frac{P_{\mathrm{signal}}}

{P_{\mathrm{error}}}

\right)

\end{equation}

where

\begin{equation}

P_{\mathrm{error}}

=

\frac{1}{L}

\sum_{k=1}^{L}

(x[k]-\hat{x}[k])^2.

\end{equation}

For the noisy signal, the error is calculated between the original and

noisy signals. For the Kalman output, the error is calculated between

the original and estimated signals.

% =========================================================

% XII. QUANTITATIVE RESULTS

% =========================================================

\section{Quantitative Results}

The results obtained from the MATLAB simulation are summarized in Table~\ref{tab:performance}.

\begin{table}[H]

\centering

\caption{Quantitative Performance Comparison}

\label{tab:performance}

\begin{tabular}{lcc}

\toprule

\textbf{Metric} & \textbf{Noisy Signal} & \textbf{Kalman Output} \\

\midrule

MSE  & 0.209776 & 0.626822 \\

RMSE & 0.458013 & 0.791721 \\

SNR  & 5.04 dB & 0.29 dB \\

\bottomrule

\end{tabular}

\end{table}

The noisy signal has an MSE of 0.209776 and an RMSE of 0.458013.

After applying the current Kalman filter configuration, the MSE

increases to 0.626822 and the RMSE increases to 0.791721.

The effective SNR of the noisy signal is 5.04 dB, while the SNR of the

Kalman output is 0.29 dB.

The SNR change is calculated as

\begin{equation}

\Delta SNR =

SNR_{\mathrm{Kalman}}

-

SNR_{\mathrm{Noisy}}.

\end{equation}

Substituting the measured values gives

\begin{equation}

\Delta SNR =

0.29 - 5.04

=

-4.75~\mathrm{dB}.

\end{equation}

Therefore, the present implementation produces an SNR change of

-4.75 dB.

% =========================================================

% XIII. DISCUSSION

% =========================================================

\section{Discussion}

The results highlight how strongly Kalman filter performance depends on the model used to describe the signal.

Kalman filters are widely used for noise reduction and state estimation, but they do not automatically improve every signal. Their performance depends on whether the state-space model describes the signal well and whether the covariance matrices are chosen appropriately.

In this experiment, the state vector is

\begin{equation}

X =

\begin{bmatrix}

\text{signal amplitude}\\

\text{rate of change}

\end{bmatrix}.

\end{equation}

This model is simple, while the test signal contains three frequencies: 100 Hz, 250 Hz, and 400 Hz. The changes in such a signal may be too complex for a model that tracks only amplitude and rate of change.

The process noise covariance matrix is

\begin{equation}

Q =

\begin{bmatrix}

0.001 & 0\\

0 & 0.001

\end{bmatrix}

\end{equation}

and the measurement noise covariance is selected from the estimated

noise power.

The values of Q and R affect the Kalman gain, which sets the balance between the model's prediction and the measured data. If the uncertainty is not represented well, the filter may place too much weight on a prediction that does not match the actual signal.

The measured results show this effect. Instead of reducing the

reconstruction error, the current configuration increases both MSE

and RMSE. Similarly, the effective SNR decreases from 5.04 dB to

0.29 dB.

These results are best understood as evidence that the filter needs a more suitable model or better tuning, not as proof that Kalman filtering is ineffective in general.

% =========================================================

% XIV. LIMITATIONS

% =========================================================

\section{Limitations}

There are a few limitations to the current implementation.

First, the state model contains only two states: signal amplitude and

rate of change. A multi-frequency sinusoidal signal may require a more

appropriate dynamic model.

Second, the process covariance matrix Q is selected using fixed

values. Different signal characteristics may require different

values.

Third, the measurement covariance R is based on the estimated noise

power. In practical systems, the measurement noise statistics may not

always be known accurately.

Fourth, the initial state and covariance are selected using simple

assumptions. The initial conditions can influence the transient

response of the filter.

Finally, the current experiment uses simulated Gaussian noise.

Real-world signals can contain non-Gaussian noise, interference,

impulsive disturbances, and time-varying noise characteristics.

% =========================================================

% XV. FUTURE WORK

% =========================================================

\section{Future Work}

The following changes could be explored in future work.

\begin{enumerate}

\item The process noise covariance matrix Q can be optimized for

the specific characteristics of the signal.

\item The measurement noise covariance R can be estimated

adaptively from the incoming measurements.

\item A higher-order state-space model can be developed to explicitly

represent the sinusoidal components of the signal.

\item Adaptive Kalman filtering can be investigated for time-varying

noise conditions.

\item The proposed algorithm can be tested using real-world sensor

signals instead of simulated Gaussian noise.

\item The Kalman filter can be compared with conventional FIR, IIR,

Wiener, and moving-average filters.

\item The implementation can be optimized for real-time execution on

an FPGA or embedded processor.

\end{enumerate}

These improvements would allow the system to be evaluated under more

realistic operating conditions.

% =========================================================

% XVI. CONCLUSION

% =========================================================

\section{Conclusion}

This project implemented a discrete-time Kalman filter in MATLAB and examined its performance on a noisy signal. The test signal combined sinusoidal components at 100 Hz, 250 Hz, and 400 Hz, and Gaussian noise was added with a target input SNR of 5 dB.

The filter used two state variables: signal amplitude and rate of change. At each sample, it predicted the state and corrected the prediction using the measurement, with the process and measurement covariance matrices controlling how the update was performed.

The results were examined using time-domain plots, frequency spectra, MSE, RMSE, and SNR. The noisy signal had an MSE of 0.209776 and an RMSE of 0.458013. For the Kalman output, these values increased to 0.626822 and 0.791721, respectively. The effective SNR also fell from 5.04 dB for the noisy signal to 0.29 dB after filtering, a decrease of 4.75 dB.

For this multi-frequency signal, the current filter settings did not reduce the noise effectively. The experiment shows that a Kalman filter's performance depends on choosing a state-space model that fits the signal and selecting suitable process and measurement noise covariance values.

Further work could focus on improving the model, tuning the covariance matrices, and testing adaptive Kalman filtering. These steps may lead to better estimates and help make the method more useful for practical signal-processing applications.

% =========================================================

% REFERENCES

% =========================================================

\begin{thebibliography}{00}

\bibitem{kalman1960}

R. E. Kalman,

``A new approach to linear filtering and prediction problems,''

\textit{Journal of Basic Engineering},

vol. 82, no. 1, pp. 35--45, 1960.

\bibitem{brown2012}

R. G. Brown and P. Y. C. Hwang,

\textit{Introduction to Random Signals and Applied Kalman Filtering},

4th ed.

Wiley, 2012.

\bibitem{maybeck1979}

P. S. Maybeck,

\textit{Stochastic Models, Estimation, and Control},

vol. 1.

Academic Press, 1979.

\bibitem{bar2001}

Y. Bar-Shalom, X. R. Li, and T. Kirubarajan,

\textit{Estimation with Applications to Tracking and Navigation}.

Wiley, 2001.

\bibitem{haykin2001}

S. Haykin,

\textit{Adaptive Filter Theory},

4th ed.

Prentice Hall, 2001.

\bibitem{oppenheim2010}

A. V. Oppenheim and R. W. Schafer,

\textit{Discrete-Time Signal Processing},

3rd ed.

Pearson, 2010.

\bibitem{welch1967}

P. D. Welch,

``The use of fast Fourier transform for the estimation of power

spectra: A method based on time averaging over short, modified

periodograms,''

\textit{IEEE Transactions on Audio and Electroacoustics},

vol. 15, no. 2, pp. 70--73, 1967.

\bibitem{matlab}

MathWorks,

``MATLAB Documentation: Signal Processing and Filtering,''

The MathWorks, Inc.

\end{thebibliography}

% =========================================================

% END DOCUMENT

% =========================================================

\end{document}